MATERIAL SAFETY DATA SHEET

Effective Date: February 2001

Supersedes:

1.  Product and manufacturer's identification

Substance Name:			SATINEX

## Grades:  120, 150, 240 & 320

Substance description:		Red-brown  coloured bar of rubber-					like consistency

Manufacturer:			Lea Manufacturing Company

PO Box 1, Anderson Works,

Tongue Lane, Fairfield,

Buxton, Derbyshire SK17 7LN

Tel:  01298-25335/6   Fax: 01298-79945

2.  Composition

OES (8hr TWA)	Content %

Aluminium oxide  (total inhalable)			      10.0			   60 - 70

(respirable)			        4.0

Gelatine    								   10 - 15

Water									   18 - 23

3.  Hazard identification

The product is NOT considered hazardous in shipping, storage or handling.  However observe precautions for the dust generated by the user.

4.  First aid

Eyes:		Flush with water for 15 minutes.  Seek medical advice

Skin:		Wash with plenty of soap and water

Inhalation:	If exposed to excessive levels of dust, remove to fresh air

Ingestion:	Drink large quantity of water - do not induce vomiting

5.	Fire fighting measures

Properties:			Flash point >190°C

Extinguishing agents:		CO2, water fog, or dry powder

Combustion products:		At high temperature, product may emit carbon monoxide and 				carbon dioxide

6.	Accidental release measures

Non hazardous, but do not wash into sewer.  Scoop up material for re-use, reclaim or proper disposal.

7.	Handling and storage

Storage temperature - ambient.

General:  Keep out of sun and away from heat sources as product may melt.  Observe all safeguards for container residue until cleaned or destroyed.

8.	Personal protection.  Exposure control

Local exhaust ventilation should be provided to maintain exposure level below the OES (Occupational exposure standard) during use.  Where LEV cannot be provided, use a suitable respirator.  Use safety glasses/goggles, rubber gloves and overalls.  Change overalls when contaminated with compound.

9.	Physical and chemical properties

Solid  Compositions

Physical aspect:				Solid grease bar

Odour:					Mild

Colour:					Red-brown

Boiling point:				>100°C

S.G.					>2.0

Vapour pressure:			N/A

Viscosity:				N/A

Solubility in water:			Partial

pH value:				Neutral

Flash point:				N/A

Melting point:				Not determined

Bursting limits:				N/A

10.	Stability and reactivity

Stable and hazardous polymerisation will not occur.  No incompatibility anticipated during normal industrial use.

11.	Toxicological information

Non-toxic but may cause eye or skin irritation.

12.	Ecological information

Avoid washing into sewer.  Biodegradable.

13.	Disposal considerations

Product in original form is non-hazardous.  Dispose of in accordance with local regulations for non-hazardous waste.

14.	Transport information

Non-hazardous.

15.  Regulatory information

Label for supply:		Not necessary

Risk phrases:			N/A

Safety phrases:			S24:  Avoid contact with skin

S25:  Avoid contact with eyes

16.	Other information

User Notes;		This information is based on the current state of our 					knowledge, and is intended to describe the product from the 				view-point of safety requirements.

Information Sources:	Health and Safety Guidance Note EH40

Occupational Exposure Limits,

Croners Reference, and our Raw Material Suppliers' Data Sheets

# WARRANTY

We warrant our goods to conform to our standard specifications.  This Warranty is in lieu of any and all other warranties or guarantees and our obligation hereunder for breach of such warranty is limited to either refund of the purchase price or replacement of said goods as we may elect.  We make no other warranties, express or implied, including those of MERCHANTABILITY and FITNESS FOR A PARTICULAR PURPOSE.

We believe that the statements technical information and recommendations contained herein are reliable, but are given without warranty or guarantee of any kind except as specified above.  Beyond that specified above, we assume no responsibility for any loss, damage or expense, direct or consequential, arising out of the use of, or inability to use, our goods.  Their quality and suitability for any particular purpose or use should be confirmed by the user's own tests.

Product will remain fit for use for a minimum of 6 months provided normal care is taken to ensure appropriate storage conditions and that any special storage conditions given on the product labels are adhered to at all times.